SUNFLOWER SOLAR PANEL CONTROLLER

An Intelligent, Autonomous Solar Tracking & Monitoring System

Competition Presentation Report  |  April 2026

# 1.  Introduction

The Sunflower Solar Panel Controller is a two-unit ESP32-based system that mimics the behaviour of a sunflower — deploying a solar panel at sunrise, tracking the sun throughout the day with a servo motor, and retracting the panel safely at sunset. A separate remote unit displays live sensor data on an OLED screen and allows the operator to manually override the servo and DC motor via four physical buttons, all over a private WiFi TCP connection that requires no internet infrastructure.

The project addresses a well-known problem in solar energy: fixed panels lose 25–40% of potential energy because they cannot follow the sun. Commercial solar trackers solve this but cost ₹40,000 or more per unit. This system achieves the same core functionality for under ₹2,500 in components, making professional-grade solar tracking accessible to small farmers, off-grid homes, and developing regions worldwide.

# 2.  System Architecture

The system consists of two independent ESP32 microcontroller units that communicate over a private WiFi network using TCP sockets.

## 2.1  Solar Controller (Device 1)

Hardware components and GPIO assignments:

| Component | Interface | GPIO Pins |
| --- | --- | --- |
| L298N Motor Driver | Digital + PWM | IN1=18, IN2=16, ENA=17 |
| Servo Motor (continuous) | PWM 50 Hz | GPIO 25 |
| Panel Voltage Sensor | ADC | GPIO 34 |
| Panel Current Sensor (ACS712) | ADC | GPIO 35 |
| Battery Voltage Sensor | ADC | GPIO 32 |
| RTC DS3231 | I²C | SDA=13, SCL=14 |

## 2.2  Remote Control Unit (Device 2)

The remote unit acts as a WiFi Access Point (192.168.4.1:5000). It hosts an OLED 128×64 dashboard showing live panel voltage, current, power, battery voltage, and the current time from the controller's RTC. Four buttons allow manual control:

| Button | Command Sent | Command on Release | Effect |
| --- | --- | --- | --- |
| Button 1 | SERVO_FWD | SERVO_STOP | Hold to rotate servo forward |
| Button 2 | SERVO_REV | SERVO_STOP | Hold to rotate servo reverse |
| Button 3 | PANEL_OPEN | MOTOR_STOP | Hold to open panel manually |
| Button 4 | PANEL_CLOSE | MOTOR_STOP | Hold to close panel manually |

# 3.  Communication Protocol

## 3.1  Controller → Remote (every 100 ms)

CSV line over TCP:

HH:MM:SS , voltage , current , power , batteryVoltage

Example:  08:42:15 , 18.34 , 2.10 , 38.51 , 12.67

## 3.2  Remote → Controller (on button press/release)

| Command | Trigger | Effect |
| --- | --- | --- |
| SERVO_FWD | Hold button 1 | Servo rotates forward — manual sun tracking |
| SERVO_REV | Hold button 2 | Servo rotates reverse — undo tracking angle |
| SERVO_STOP | Release button 1 or 2 | Servo stops immediately |
| PANEL_OPEN | Hold button 3 | Motor opens panel (ignored if not IDLE) |
| PANEL_CLOSE | Hold button 4 | Motor closes panel (ignored if not IDLE) |
| MOTOR_STOP | Release button 3 or 4 | Motor stops immediately on release |
| RESET | Emergency command | Halts everything, clears all state flags |

# 4.  Key Technical Features

## 4.1  Non-Blocking Architecture

Main loop runs every 10 ms — never uses delay(). All operations use millis() timers.

Motor, servo, WiFi reconnect, and sensors all run concurrently without pausing each other.

WiFi client.connect() is only attempted when motorState == MOTOR_IDLE to prevent blocking the PWM ramp (connect() can stall the CPU for up to 500ms).

RTC is read over I²C every 1 second and cached — avoids 100× I²C overhead on every 10ms loop iteration.

## 4.2  Motor State Machine

States: MOTOR_IDLE → MOTOR_OPENING / MOTOR_CLOSING / MOTOR_MANUAL → MOTOR_IDLE

New commands (schedule or manual) are only accepted when motorState == MOTOR_IDLE, preventing double-trigger.

The opened/closed flags are set after the motor physically finishes its run — not when the command fires — reflecting real-world panel position.

Emergency RESET instantly calls hardBrake(), sets motorState = MOTOR_IDLE, and clears all timing variables.

## 4.3  PWM Soft-Ramp Motor Control

Custom MotorL298N library ramps PWM from 0% → 100% over approximately 5 seconds (1% per 50ms by default).

A 30% kickstart pulse is applied immediately on setSpeed() to overcome static friction before ramping.

hardBrake() instantly cuts all PWM and direction pins — used for emergency RESET.

Smooth ramp significantly reduces mechanical shock, extending the life of the gearbox, belt, and panel mounting hardware.

## 4.4  Automatic Current Sensor Calibration

On every boot, 500 ADC samples are averaged over 1 second with no load connected.

The resulting mean voltage is stored as ACS_OFFSET — the zero-current reference.

This eliminates offset drift caused by temperature changes and supply voltage variation.

No manual potentiometer or trim adjustment is required — the system self-configures each startup.

## 4.5  Servo Sun Tracking

Auto-tracking fires every 15 minutes while the panel is open (configurable via SERVO_TRACK_INTERVAL).

Duration is mathematically calculated from SERVO_RPM and SERVO_TRACK_REVS — not guessed.

Priority order: Manual command > Auto-track > Stop. Manual override pauses auto-tracking instantly.

Auto-tracking is cancelled immediately when the panel closes at end of day.

# 5.  Large Scale Implementation

## 5.1  Hardware Scaling

Replace the single ESP32 controller with a Raspberry Pi or industrial PLC as a central hub managing hundreds of panel units over a network.

The L298N motor driver is replaced with 3-phase motor controllers for the larger DC or AC actuators used in commercial trackers.

Stepper motors with quadrature encoders replace the continuous-rotation servo, providing precise angular position control and eliminating open-loop drift.

Industrial-grade CT clamps rated for 100A+ replace the ACS712 modules for high-current installations.

The OLED display is replaced by a web-based Grafana or React dashboard showing fleet-wide metrics on a map, with per-panel status indicators.

Dual-axis tracking: a second motor added for elevation angle control increases annual yield by a further 10–15% beyond single-axis.

## 5.2  Network and Software Scaling

Replace direct TCP with an MQTT broker (Mosquitto). Each panel publishes sensor data to its own topic; a central server subscribes to all topics simultaneously.

OTA (Over-the-Air) firmware updates push new code to all panels in a field without physical access to each unit.

Time-series database (InfluxDB or TimescaleDB) stores all sensor readings for historical analysis, fault detection, and energy yield reporting.

A REST API allows any monitoring tool (mobile app, web browser, SCADA system) to query panel status, issue commands, and update schedules.

GPS-based sun angle calculation using the NOAA solar position algorithm computes optimal panel angle from latitude, longitude, and UTC time — eliminating the fixed 15-minute interval assumption.

Cloud backup with automated daily energy yield reports delivered via email or SMS to the owner.

## 5.3  Physical Deployment

Panels mounted on single-axis tracker rails (east-west rotation) driven by the DC motor — the most common commercial solar tracker design.

Wind speed sensor auto-retracts all panels when gusts exceed a configurable safe threshold, preventing storm damage.

IP65-rated weatherproof enclosures for all electronics with cable glands; surge protection and reverse-polarity protection on all power inputs.

Modular wiring harness — each panel unit is identical and plug-and-play, reducing installation time and training requirements.

Staggered motor start schedules prevent current surges when all 100+ panels attempt to open simultaneously at sunrise.

# 6.  System Improvements

## 6.1  Precision Sun Tracking

Add an LDR (light-dependent resistor) differential pair — the servo rotates until both sensors read equal illumination, meaning the panel is facing the sun directly. Replaces the open-loop timed pulse.

Closed-loop position control: rotary encoder on the servo shaft confirms actual angular position and enables the controller to command precise angles rather than timed rotations.

MPPT algorithm (Maximum Power Point Tracking): continuously adjust panel angle to maximise real-time power output, not just geometric alignment with the sun. Accounts for cloud diffusion and partial shading.

Add an elevation axis motor for full 2-axis tracking — adds 10–15% annual energy yield over single-axis alone.

## 6.2  Reliability and Safety

Motor stall detection: add a dedicated current sensor on the motor supply line. A current spike without movement triggers an immediate halt and fault alert.

Panel position encoder: know the exact open angle at all times. Prevents over-driving the motor if the panel is mechanically obstructed.

ESP32 hardware watchdog timer: auto-reboots the system if the firmware hangs, ensuring the panel is never left in an unknown state.

Redundant time source: sync from NTP when WiFi is available; fall back to DS3231 RTC when offline. RTC drift corrected automatically after each NTP sync.

Surge protection and TVS diodes on all ADC input lines to prevent sensor damage from lightning or motor back-EMF.

## 6.3  Energy Intelligence

State-of-charge calculation: derive battery percentage from the discharge voltage curve rather than relying on raw voltage alone.

Watt-hour accumulation: log daily energy yield (kWh) to flash memory for comparison against historical and predicted values.

Low battery protection: automatically retract the panel and enter sleep mode when battery voltage drops critically low, protecting the battery from deep discharge.

Efficiency monitoring: compare actual yield to expected yield based on solar irradiance models; alert when efficiency drops more than 10% below threshold (indicating soiling or shading).

Panel temperature sensor: derate power estimate on hot days (approximately 0.3% per °C above 25°C for silicon cells).

## 6.4  User Interface

Replace the OLED with a colour TFT touchscreen showing scrollable voltage, current, and power graphs with time-axis.

Mobile app via Bluetooth or WiFi for configuration: set open/close times, tracking interval, and RPM calibration without touching firmware.

Push notifications when power drops below expected threshold or a fault is detected — delivered via SMS, WhatsApp, or email.

Web interface with historical charts, exportable CSV data, and remote schedule configuration accessible from any browser.

Voice alerts via a small buzzer: distinct tones for open complete, close complete, and fault events.

# 7.  Future Benefits and Applications

## 7.1  Energy Independence

Single-axis tracking increases annual energy yield by 25–40% over a fixed panel installation.

Adding a second (elevation) axis increases yield by a further 10–15%, for a total gain of up to 55% over fixed.

More energy from the same number of panels reduces the total hardware cost and land area required for a given power target.

Longer battery cycle life: the tracker maximises energy extraction during peak sun hours, reducing the number of days the battery must supply energy without solar recharge.

## 7.2  Agrivoltaics (Agriculture + Solar)

Tilted solar panels allow morning and evening light to reach crops underneath while blocking harsh midday sun.

Crops grown under tracked panels benefit from reduced heat stress, with studies showing up to 60% less water consumption in some climates.

Dual income stream: farmers generate and sell electricity while continuing to grow crops on the same land.

This system's automated tilt angle control makes it a ready-to-use foundation for agrivoltaic installations at the village level.

## 7.3  Smart Grid Integration

Real-time power data at 100ms resolution feeds directly into grid balancing and demand-response systems.

When panels report surplus generation, smart switches can automatically divert energy to water heating, EV charging, or grid export.

Predictive generation forecasting: historical tracking data combined with weather APIs helps grid operators plan backup generation capacity.

Micro-grid: a cluster of these units with a shared battery bank can power an entire village with automatic load balancing.

## 7.4  Developing World and Disaster Relief

No internet required — the two ESP32s form their own private WiFi network, deployable in remote areas with zero infrastructure.

Total BOM cost under ₹2,500 (~$30) makes professional solar tracking accessible to smallholder farmers globally.

Can be assembled with basic tools and locally available modules — no specialist manufacturing required.

Portable solar tracker + battery provides uninterrupted power for medical equipment, water pumps, and communications in disaster zones.

## 7.5  Environmental Impact

| Scale | Daily Energy Gain vs Fixed | Annual CO₂ Offset |
| --- | --- | --- |
| 1 rooftop panel (300W) | +~100 Wh/day | ~15 kg CO₂/year |
| 100-panel farm (30 kW) | +~10 kWh/day | ~1,500 kg CO₂/year |
| Utility-scale (1 MW) | +~350 kWh/day | ~50,000 kg CO₂/year |

Based on 35% improvement from single-axis tracking, 300 sunny days/year, and 0.4 kg CO₂/kWh grid average (IEA 2024).

# 8.  Competitive Advantages

No Internet Required: The private WiFi network between the two ESP32s works in fields, deserts, and rooftops without any infrastructure. Unlike cloud-dependent systems, it cannot be disrupted by a server outage.

Custom PWM Motor Library: Soft-ramp 0→90% over 5 seconds with kickstart prevents gear shock, motor burnout, and belt snap. No commercial tracker at this price offers configurable ramp control.

Self-Calibrating Sensors: ACS712 zero offset auto-calibrated on every boot with 500 samples — compensates for temperature drift and eliminates manual potentiometer tuning.

State-Machine Safety: Motor only accepts new commands when IDLE. Flags reflect physical panel state, not command state. Single RESET halts everything to a known safe state.

100ms Live Data Stream: 10 readings per second is sufficient to detect transient shadows, partial shading, and wiring faults — far beyond the 1-second resolution of most hobby systems.

Extensible Protocol: Adding a new remote command requires one line in the buttons[] array. No other code changes needed — engineers can extend the system in minutes.

Under ₹2,500 Total Cost: Commercial solar trackers start at ₹40,000+. This system delivers the core functionality for 94% less, making it viable for mass deployment in developing markets.

# 9.  Bill of Materials

| Component | Qty | Unit Price | Total | Purpose |
| --- | --- | --- | --- | --- |
| ESP32 Dev Board | 2 | ₹400 | ₹800 | Controller + Remote unit |
| L298N Motor Driver | 1 | ₹150 | ₹150 | DC motor control |
| DC Gear Motor (12V) | 1 | ₹350 | ₹350 | Panel open/close |
| Continuous Rotation Servo | 1 | ₹280 | ₹280 | Sun tracking |
| ACS712-30A Current Sensor | 1 | ₹120 | ₹120 | Panel current |
| Voltage Divider Module | 2 | ₹45 | ₹90 | Panel + battery voltage |
| DS3231 RTC Module | 1 | ₹180 | ₹180 | Schedule timekeeping |
| SSD1306 OLED 128×64 | 1 | ₹220 | ₹220 | Live dashboard display |
| Push Buttons (pack) | 1 | ₹50 | ₹50 | Manual control buttons |
| Misc (wires, PCB) | 1 | ₹200 | ₹200 | Wiring and prototyping |
| TOTAL |  |  | ₹2,440 |  |

# 10.  Conclusion

The Sunflower Solar Panel Controller demonstrates that intelligent, autonomous solar tracking is not a luxury reserved for large utility companies. With under ₹2,500 of components and carefully engineered firmware, it is possible to build a system that:

Increases annual solar energy yield by 25–40% through automated sun tracking.

Monitors real-time panel voltage, current, and power at 100ms resolution.

Provides a wireless OLED dashboard with live data and manual override — no internet required.

Uses a robust state-machine architecture that is safe, non-blocking, and extensible.

Scales directly to large solar farms via MQTT, cloud databases, and OTA firmware updates.

Saves up to 50,000 kg of CO₂ per year per MW of solar capacity compared to fixed installations.

"The future of solar is not just bigger panels — it is smarter panels that follow the sun, report their health, and respond to their environment automatically."